National Instruments NI-488 MS-DOS and Windows Handlers

Revision E.7

Copyright 1991  National Instruments Corporation.

All Rights Reserved.

Revisions E.5 and later of the NI-488 handlers contain several new

functions not listed in the "NI-488 Software Reference Manual."  In

addition, several existing NI-488 functions have been modified in this

release. This file first describes the changes made to existing NI-488

functions and then lists the new NI-488 functions.

===========================================================================

CHANGES TO NI-488

-----------------

1.  The following I/O transfer functions now accept a 32-bit transfer

count value:

ibcmd   ibcmda   ibrd   ibrda   ibwrt   ibwrta

However, because BASICA limits integers to 16-bits, the BASICA

language interface has not been expanded to 32-bit transfer counts.

All of the transfer functions still accept 16-bit counts to support

existing applications.

The global variable 'ibcnt' is still a 16-bit number.  To access the

full 32-bit transfer count, a new global variable named ibcntl has

been added.

2.  To improve clarity, the description of asynchronous I/O for the

ibrda, ibwrta, and ibcmda functions is expanded.

By using one of three asynchronous I/O calls (ibcmda, ibrda, or ibwrta),

an application can perform other functions (non-GPIB functions) while

processing the I/O.  Once the asynchronous I/O call has been initiated,

further GPIB calls involving the device or access board are not allowed

until the I/O has completed and the GPIB driver and the application have

been resynchronized.

Resynchronization can be accomplished by using one of the following

three functions:

NOTE:   Resynchronization is only successful if the ibsta returned

contains CMPL.

+   ibwait (mask contains CMPL)  -  The driver and application are

synchronized.

+   ibstop                       -  The asynchronous I/O is canceled,

and the driver and application

are synchronized.

+   ibonl                        -  The asynchronous I/O is canceled,

the interface has been reset,

and the driver and application are

synchronized.

The only other GPIB call that you can use during asynchronous I/O is the

ibwait function (mask is arbitrary).  Any other GPIB call involving the

device or access board returns the EOIP error.

===========================================================================

NEW NI-488 FUNCTIONS

--------------------

The following is a list of the new NI-488 functions:

ibconfig    iblines    ibrdkey    ibwrtkey

The remainder of this file contains a detailed description of each new

NI-488 function with examples.

==============

IBCONFIG

==============

Purpose:        Change the handler configuration parameters.

Format:

BASICA

CALL ibconfig (ud%, option%, value%)

QuickBASIC

CALL ibconfig (ud%, option%, value%)

or

ilconfig (ud%, option%, value%)

C

int ibconfig (int ud, unsigned int option, unsigned int value)

'ud' specifies a GPIB interface board or a device.  'option' is used to

select the item in the handler to be configured.  The configurable item is

set to the contents of 'value'.  The previous contents of the configurable

item is returned in iberr.  If 'ud' is a GPIB interface board descriptor,

'option' contains one of the values shown in Table 1.  If 'ud' is a device

descriptor, 'option' has one of the values shown in Table 2.

Table 1:  Board Configuration Options

=====================================

Option    Description

------    -----------

1       Primary Address.  value is the new primary address of the

GPIB interface board  (0 through 30).  See IBPAD.

2       Secondary Address.  value is the new secondary address of

the board (0, 96 through 126).  See IBSAD.

3       Timeout Value.  value is the new timeout value of the

board (0 through 17).  See IBTMO.

4       Enable/disable END message on write operations.  value is

the new EOT mode (0 = no END, non-zero = send END with

last byte).  See IBEOT.

5       Parallel Poll Configure.  value is the parallel poll

configure byte (0, 96 through 126).  See IBPPC.

7       Enable/disable Automatic Serial Polling.  If value is zero

(0), Autopolling is disabled.  If value is non-zero,

Autopolling is enabled.

8       Use/do not use the NI-488 CIC protocol.  If value is zero

(0), do not use the CIC protocol.  If value is non-zero, use

the CIC protocol.  The CIC protocol is a scheme whereby

control may be passed between devices when a request for

service occurs and the serial poll response byte equals

hex 42.  This protocol is off by default.

9       Enable/disable hardware interrupts.  If value is zero (0),

disable GPIB interface board interrupts.  If value is non-

zero, enable GPIB interface board interrupts.  See the

description of the IBCONF utility program in Section two

of the "NI-488 MS-DOS Software Reference Manual".

10      Request or release System Control.  If value is zero (0),

the GPIB board will not be the System Controller. Functions

requiring System Controller capability are disallowed.  If

value is non-zero, the GPIB board will be the System

Controller.  Functions requiring System Controller capability

are allowed.  See IBRSC.

11      Assert/unassert REN.  If value is non-zero, the IEEE-488

Remote Enable (REN) signal is asserted.  If value is zero

(0), REN is unasserted.  See IBSRE.

12      Terminate read when End-Of-String (EOS) character is

detected.  If value is non-zero, read functions are

terminated when the EOS character is detected in the data

stream.  If value is zero, EOS detection is disabled.  See

IBEOS.

13      Assert EOI when sending EOS character.  If value is zero

(0), do not send EOI with EOS.  If value is non-zero, send

EOI with EOS.  See IBEOS.

14      Use 7- or 8-bit EOS comparison.  If value is zero, use low-order

7 bits of EOS character for comparison.  If value is non-zero,

use 8 bits.  See IBEOS.

15      End-Of-String (EOS) character.  value is the new

EOS character of the board (8 bits).  See IBEOS.

16      Parallel Poll remote/local configuration.  If value is zero,

the GPIB interface board uses IEEE-488 Parallel Poll (PP)

interface function subset PP1 (remote configuration by

external Controller).  If value is non-zero, the board uses

PP subset PP2 (local configuration from your application

program:  value is used as the local poll enable [lpe]

message).  See IBPPC.

17      IEEE-488 bus handshake timing.  If value is one (1),

normal timing is used for the IEEE-488 Source Handshake

T1 delay (2 msec).  If value is two (2), high-speed timing

is used for T1 (500 nsec).

18      Enable/disable direct memory access (DMA) transfers.  If

value is zero (0), disable GPIB interface board DMA

transfers.  If value is non-zero, enable GPIB interface board

DMA transfers.  See the description of the IBCONF utility

program.

19      Byte swapping on ibrd.  If value is one (1), pairs of bytes

read off the bus are swapped before storing them in the ibrd

buffer.  The transfer count must be even or ECAP will be

returned.  In this case, the last two bytes of the buffer

will be invalid.  If ECAP is returned and your buffer begins

on an odd address, start the buffer on an even address.

If value is zero (0), byte swapping on ibrd is disabled.

20      Byte swapping on ibwrt.  If value is one (1), pairs of

bytes are swapped before they are written from the user's

buffer to the bus.  The transfer count must be even or ECAP

will be returned.  In some cases, the address of the buffer

must be even.  If ECAP is returned and your buffer begins on

an odd address, start the buffer on an even address.  If

value is zero (0), byte swapping on ibwrt is disabled.

Table 2:  Device Configuration Options

======================================

Option    Description

------    -----------

1       Primary Address.  value is the new primary address of the

device (0 through 30).  See IBPAD.

2       Secondary Address.  value is the new secondary

address of the device (0, 96 through 126).  See IBSAD.

3       Timeout Value.  value is the new timeout value of the

device (0 through 17).  See IBTMO.

4       Enable/disable END message on write operations.  value is

the new EOT mode (0 = no END, non-zero = send END with

last byte).  See IBEOT.

6       Repeat Addressing.  If value is zero (0), disable repeat

addressing.  If value is non-zero (1), enable repeat

addressing.  See the description of the IBCONF utility

program in Section two of the "NI-488 MS-DOS Software

Reference Manual".

12      Terminate read when End-Of-String (EOS) character is

detected from this device.  If value is non-zero, read

functions are terminated when the EOS character is detected

in the data stream received from the device.  If value is

zero, EOS detection is disabled.  See IBEOS.

13      Assert EOI when sending EOS character to this device.  If

value is zero (0), do not send EOI with EOS.  If value is

non-zero, send EOI with EOS.  See IBEOS.

14      Use 7- or 8-bit EOS comparison.  If value is zero, use low-

order 7 bits of EOS character for comparison.  If value is

non-zero, use 8 bits.  See IBEOS.

15      End-Of-String (EOS) character.  value is the new EOS

character (8 bits) to use with this device.  See IBEOS.

19      Byte swapping on ibrd.  If value is one (1), pairs of bytes

read off the bus are swapped before storing them in the ibrd

buffer.  The transfer count must be even or ECAP will be

returned.  In this case, the last two bytes of the buffer

will be invalid.  If ECAP is returned and your buffer begins

on an odd address, start the buffer on an even address.

If value is zero (0), byte swapping on ibrd is disabled.

20      Byte swapping on ibwrt.  If value is one (1), pairs of

bytes are swapped before they are written from the user's

buffer to the bus.  The transfer count must be even or ECAP

will be returned.  In some cases, the address of the buffer

must be even.  If ECAP is returned and your buffer begins on

an odd address, start the buffer on an even address.  If

value is zero (0), byte swapping on ibwrt is disabled.

Device Function Examples for IBCONFIG

=====================================

Set up various configurable parameters in preparation for a device read.

BASICA/QuickBASIC

-----------------

100  dev$ = "dev1"

110  CALL ibfind (dev$, dev1%)

120  REM Enable repeat addressing

130  option% = 6 : value% = 1

140  CALL ibconfig (dev1%, option%, value%)

150  REM Set linefeed as the EOS character

160  option% = 15 : value% = 10

170  CALL ibconfig (dev1%, option%, value%)

180  REM Use 7-bit comparison for EOS character

190  option% = 14 : value% = 0

200  CALL ibconfig (dev1%, option%, value%)

210  REM Terminate reads on EOS

220  option% = 12 : value% = 1

230  CALL ibconfig (dev1%, option%, value%)

QuickBASIC

----------

' open device

CALL ibfind ("dev1", dev1%)

' Enable repeat addressing

CALL ibconfig (dev1%, 6, 1)

' Set linefeed as the EOS character

CALL ibconfig (dev1%, 15, 10)

' Use 7-bit comparison for EOS character

CALL ibconfig (dev1%, 14, 0)

' Terminate reads on EOS

CALL ibconfig (dev1%, 12, 1)

C

-

/* open device */

dev1 = ibfind ("dev1");

/* Enable repeat addressing */

ibconfig (dev1%, 6, 1)

/* Set linefeed as the EOS character */

ibconfig (dev1%, 15, 10)

/* Use 7-bit comparison for EOS character */

ibconfig (dev1%, 14, 0)

/* Terminate reads on EOS */

ibconfig (dev1%, 12, 1)

Board Function Examples for IBCONFIG

====================================

1.  Set up various configurable parameters in preparation for a board read.

BASICA/QuickBASIC

-----------------

100  gpib$ = "gpib0"

110  CALL ibfind (gpib$, gpib0%)

120  REM Enable DMA transfers

130  option% = 18 : value% = 1

140  CALL ibconfig (gpib0%, option%, value%)

150  REM Turn off Autopolling

160  option% = 7 : value% = 0

170  CALL ibconfig (gpib0%, option%, value%)

180  REM Turn on interrupts

190  option% = 9 : value% = 1

200  CALL ibconfig (gpib0%, option%, value%)

QuickBASIC

----------

' open GPIB interface board

CALL ibfind ("gpib0", gpib0%)

' Enable DMA transfers

CALL ibconfig (gpib0%, 18, 1)

' Turn off Autopolling

CALL ibconfig (gpib0%, 7, 0)

' Turn on interrupts

CALL ibconfig (gpib0%, 9, 1)

C

-

/* open GPIB interface board

*/

gpib0 = ibfind ("gpib0");

/* Enable DMA transfers

*/

ibconfig (gpib0, IBC_DMA, 1);

/* Turn off Autopolling

*/

ibconfig (gpib0%, IBC_AUTOPOLL, 0);

/* Turn on interrupts

*/

ibconfig (gpib0%, IBC_IRQ, 1);

2.  Enable automatic byte swapping of binary integer data.

BASICA/QuickBASIC

-----------------

50      DIM array% (500)

100     REM Read in unswapped header data

101     REM (10 bytes).

102     header$ = space$(10)

103     CALL ibrd (ud%, header$)

104     REM Arrange for byte swapping.

105     option% = 19 : value% = 1

106     CALL ibconfig (ud%, option%, value%)

107     REM Read 1,000 bytes with automatic

108     REM swapping.

109     count% = 1000

110     CALL ibrdi (ud%, array% (0), count%)

111     REM Disable swapping for subsequent reads.

112     value% = 0

113     CALL ibconfig (ud%, option%, value%)

QuickBASIC

----------

DIM array% (500)

' Read in unswapped header data (10 bytes).

header$ = space$(10)

sta% = ilrd (ud%, header$, 10)

' Arrange for byte swapping.

sta% = ilconfig (ud%, 19, 1)

' Read 1,000 bytes with automatic swapping.

sta% = ilrdi (ud%, array% (), 1000)

' Disable swapping for subsequent reads.

sta% = ilconfig (ud%, 19, 0)

C

-

int array [500];

char header [10];

/* Read in unswapped header data.

*/

ibrd (ud, header, 10);

/* Arrange for byte swapping.

*/

ibconfig (ud, 19, 1);

/* Read 1,000 bytes with automatic swapping.

*/

ibrdi (ud, array, 1000);

/* Disable swapping for subsequent reads.

*/

ibconfig (ud, 19, 0);

---------------------------------------------------------------------------

=============

IBLINES

=============

Purpose:        Return the status of the GPIB control lines.

Format:

BASICA

CALL iblines (ud%, clines%)

QuickBASIC

CALL iblines (ud%, clines%)

or

illines (ud%, clines%)

C

int iblines (int ud, int *clines)

'ud' is a board descriptor.  A "valid" mask is returned along with the

GPIB control line state information in 'clines'.  The low-order byte (bits

0 through 7) of 'clines' contains a mask indicating the ability of the

GPIB interface board to sense the status of each GPIB control line.  The

upper byte (bits 8 through 15) contains the GPIB control line state

information.  The pattern of each byte is as follows:

7       6       5       4       3       2       1      0

-------------------------------------------------------------

EOI     ATN     SRQ     REN     IFC     NRFD    NDAC    DAV

To determine if a GPIB control line is asserted, first check the

appropriate bit in the lower byte to determine if the line can be

monitored.  If the bit can be monitored (indicated by a 1 in the

appropriate bit position), check the corresponding bit in the upper byte.

If the bit is set (1), the corresponding control line is asserted. If the

bit is clear (0), the control line is unasserted.

IBLINES Example

---------------

Test for Remote Enable (REN).

BASICA/QuickBASIC

-----------------

100 CALL ibfind ("GPIB0", gpib0%)

110 CALL iblines (gpib0%, clines%)

120 REN% = clines% AND &H10

130 IF REN% <> &H10 THEN GOTO 800

140 REN% = clines% AND ((clines%/256) AND &H10)

150 IF REN% <> &H10 THEN GOTO 900

160 PRINT "REN is asserted!" : STOP

800 PRINT "GPIB board is unable to monitor REN."

810 STOP

900 PRINT "REN is not asserted!" : STOP

920 END

QuickBASIC

----------

gpib0% = ilfind ("GPIB0")

IF gpib0% < 0 THEN GOTO ERROR

CALL iblines (gpib0%, clines%)

IF ibsta% < 0 THEN GOTO ERROR

REN% = clines% AND &H10

IF REN% <> &H10 THEN GOTO BRDERR

REN% = REN% AND (clines%/256)

IF REN% <> &H10 THEN GOTO UNASSERTED

PRINT "REN is asserted." : STOP

ERROR:

PRINT "GPIB Handler error." : STOP

BRDERR:

PRINT "GPIB board is unable to monitor REN!"

STOP

UNASSERTED:

PRINT "REN is not asserted." : STOP

END

C

-

main ()

{

int clines;

int gpib0;

if ((gpib0 = ibfind ("GPIB0")) < 0) error();

if ((ibsta = iblines (gpib0, &clines)) < 0) error();

if (!(clines & 0x10)) {

printf("GPIB board can't monitor REN!");

exit();

}

if (clines & 0x1000) {

printf("REN is asserted.");

exit();

}

printf("REN is not asserted.");

}

---------------------------------------------------------------------------